Machine Properties: NC Data for CAM common parameters

Immeuble le Méliès

261, rue de Paris

93 556 MONTREUIL Cedex

Tél. : +33 (0)1 43 60 25 00

Fax : +33 (0)1 43 60 25 01

http://www.ncsimul.com

|  | Benoit Souvestre |
| --- | --- |

# History

| Révision | Date | Auteur | Action | Validation |
| --- | --- | --- | --- | --- |
| 1.0 | 25 Aug 2016 | BSO | Document Creation & PrgID function description |  |
| 1.1 | 30 Aug 2016 | JDU | Toolpath splitting function description |  |
| 2.0 | 03 Oct 2016 | JDU | Documentation for version 10.2.1 |  |
| 2.1 | 04 Oct 2016 | JDU | Sections about end of working steps on codes added |  |
| 2.2 | 29 Nov 2016 | BSO | 4CAM strategy description & Automatic WS open on codes |  |
| 2.3 | 11 Jan 2016 | BSO | Examples |  |
| 2.4 | 12 Jan 2016 | JDU | Added M140 in Heidenhain retracts |  |

Table of Contents

1.	History	2

2.	Context	4

3.	Split	5

a.	Prg from 4CAM	5

b.	Trajectories naming	6

c.	Splitting strategy choice	7

i.	None (Strategy 0)	8

ii.	One sequence per tool call (strategy 1)	8

iii.	One sequence per rotary axis orientation (strategy 2)	8

iv.	One sequence per specific comment (strategy 3)	9

v.	Others (Strategy 1, 2 & 3)	10

d.	Management of the subprograms for splitting	10

i.	Ignore the Memory	10

ii.	Sup program to ignore	11

e.	Stopping a working step on retract moves	12

i.	Siemens 840D	12

ii.	Heidenhain iTNC 530	12

iii.	Fanuc (Milling)	12

iv.	Num 1060F	12

v.	Others (in case you included the splitting)	13

f.	Customize the end of working steps	13

b.	Examples	13

i.	NCsimul creates Empty WS	13

4.	PrgId	13

a.	Settings	13

5.	Debug	14

a.	Check Splitting	14

b.	Debug PrgId	14

6.	Table des illustrations	16

# Context

This document is mainly for internal use. The goal is to have a short description of “NC Data for Cam” section and configuration.

Figure 1: NC Data for Cam

# Split

This function will allow you to choose the strategy which strategy the machine will have in order to split the toolpath that will get transferred to NCSIMUL CAM.

## Prg from 4CAM

Figure 2: Prg from 4CAM in the Machine Properties

The option Prg from 4CAM will use the comments (beginning by “NCS-START : “ and by “NCS-END : “) generated by 4CAM in order to split the toolpath into working steps. If you choose this option, it overrides every other option and strategy that are chosen in the machine properties (no automatic open on canned cycle [see also Others (Strategy 1, 2 & 3)], nor automatic end on retract move [see also Stopping a working step on retract moves]).

The same option is available in the Initialization function, allowing to change the splitter settings for the current project.

Figure 3: Prg from 4CAM in the Initialization function

## Trajectories naming

Figure 4: Trajectories naming in the machine properties

This option allows to name the working step using the last captured comment by the splitter.
If the option is unticked, the working step will be automatically named: “Sequence xx Tyy”

The same option is available in the Initialization function, allowing to change the splitter settings for the current project.

Figure 5: Trajectories naming in the Initialization function

## Splitting strategy choice

Figure 6: Parameter to choose the splitting strategy in the machine properties

The same option is available in the Initialization function, allowing to change the splitter settings for the current project.

Figure 7: Parameter to choose the splitting strategy in the machine properties

There are 4 choices for choosing a strategy that will split the toolpath.

Figure 8: Help for the strategy choice

### None (Strategy 0)

There won’t be any splitting, the machine is not intended to transfer job to NCSIMUL CAM

### One sequence per tool call (strategy 1)

The toolpath will be split between every tool change.

### One sequence per rotary axis orientation (strategy 2)

The tool path will be split each time a rotary axis is moved in a rapid motion.
If you choose this strategy: you need to input the CNU number of the rotary axes that will impact the toolpath trajectory, and to separate them with a ‘;’.

If you use the strategy 2 from the initialization function: you can change which rotary axes will be used for splitting. Don’t forget to fill in the rotary axes you choose if the strategy 2 is the one chosen in your initialization function.

Figure 9: Example of a setting with the strategy 2

### One sequence per specific comment (strategy 3)

Fill in the words contained in the comments that will be used for splitting your toolpath. Each time a word (or combination of words) is found in a comment, the toolpath is split. Use (Ctrl+Space Bar) in order to type the next element in the list.

Figure 10: Set the splitting comments up

The SPLIT_COMMENT.txt from version 10.2.0 is still compatible.

BEGIN_SPLIT_COMMENT

# List of words (or combination of words) in comments that should be used to split the comments

# File should always begin with BEGIN_SPLIT_COMMENT

# One word/combination of words per line

Drilling

Machining

END_SPLIT_COMMENT

# File should always end with END_SPLIT_COMMENT

Between the BEGIN_SPLIT_COMMENT line and the END_SPLIT_COMMENT one, you need to enter all the words in comments that will split the toolpath (here “Drilling” and “Machining”).

In the initialization function, you can find an option to ignore the comments containing a word. Write all the words (or combination of words) that the comment analysis for splitting will ignore.
If the comment also contain a word used for splitting this comment will be overlooked.

Figure 11: Comment to ignore in the initialization function

### Others (Strategy 1, 2 & 3)

When no sequence is active, but some specific NC codes for machining (like canned cycles), a sequence is automatically created.

The sequence is ended in a normal way (see related topic: Stopping a working step on retract moves)

List of NC Codes:

| Fanuc (Milling) | Heidenhain iTNC 530 | Num 1060F | Siemens 840D |
| --- | --- | --- | --- |
| G73, G76, G81, G82, G83, G84, G85, G86, G88, G89 | CYCLE DEF 200, 201, 202, 203, 207 | G81, G82, G83, G84, G85 | CYCLE73, CYCLE76, CYCLE81, CYCLE82, CYCLE83, CYCLE84, CYCLE85, CYCLE86, CYCLE88, CYCLE89 |

## Management of the subprograms for splitting

### Ignore the Memory

Figure 12: Ignore the Memory in the machine properties

If ticked, this splitting process will overlook all the sub programs contained in the memory directory of the machine.

The same option is available in the Initialization function, allowing to change the splitter settings for the current project.

Figure 13:  Ignore the Memory in the initialization function

### Sup program to ignore

Figure 14: sub programs to ignore in the machine properties

Write all the names and extensions of the programs you want the splitting process to overlook. Use (Ctrl+Space Bar) in order to type the next element in the list.

Figure 15: Sup programs to ignore in the initialization function

You can add subprograms in the initialization function: write all the names and extensions of the programs you want the splitting process to overlook, separated with a ‘;’.

## Stopping a working step on retract moves

Working steps are also ended on retract moves. (Moves programmed in machine reference or with a special position. A new working step will be started after one move in one linear axis programmed with the programmed reference.

Here is the list of codes on which we stop working steps:

### Siemens 840D

G53, G75, G153, SUPA

### Heidenhain iTNC 530

M91,M92,M140

### Fanuc (Milling)

G53, G28, G30

### Num 1060F

G52,G53

### Others (in case you included the splitting)

Whenever a move is programmed and MACHORG==0

## Customize the end of working steps

If you need to add codes to stop working step (for example here: G49) in your specific controller.

MACRO=INITPRG,APPEND
	Add_Call_On_SCode(1,49,"MAC_SPLIT_TRAJ_BREAK_ON_RETRACT()","BEFORE_CODE")
	Set_Call_On_SCode(1,49,Cnst(On))
ENDMACRO

If you are using the Heidenhain controller, don’t forget to double your code for each program context (ISO and Cleartext):

MACRO=INITPRG,APPEND

WithProgramContext(1)
	Add_Call_On_SCode(1,49,"MAC_SPLIT_TRAJ_BREAK_ON_RETRACT()","BEFORE_CODE")
	Set_Call_On_SCode(1,49,Cnst(On))

WithProgramContext(2)
	Add_Call_On_SCode(1,49,"MAC_SPLIT_TRAJ_BREAK_ON_RETRACT()","BEFORE_CODE")
	Set_Call_On_SCode(1,49,Cnst(On))
ENDMACRO

## Examples

.

### NCsimul creates Empty WS

This example will show you an extract of ISO code for a 3 axis machine, with Heidenhain Tnc530 controller. The option selected to retrieve ISO code is “N°1: 1 sequence by tool”.

With the following ISO code we notice empty Working step:

Figure 16 : Example of empty Ws

### NCSimul keep

# PrgId

The default behavior of this function will catch every comments (within the limit of 20) in the main program before the first tool change and save them inside the PrgID function.

## Settings

You can enter a default start/end tag to catch PrgID inside machine properties.

Figure 17: Begin / End tag

These setting can be overwritten with the initialization function.

Figure 18: Begin/End tag modification

# Debug

## Check Splitting

When checked, this option will display error messages on following events.

- Start of a sequence

- End of a sequence

## Debug PrgId

When checked this option will display error messages on following events:

- Setting for PrgID start / end tag

- Each time a comment is saved inside the PrgId function (value and index)

- When a start / end tag is found inside a comment

- When the PrgId function is created

# Table des illustrations

Figure 1: NC Data for Cam	4

Figure 2: Prg from 4CAM in the Machine Properties	5

Figure 3: Prg from 4CAM in the Initialization function	6

Figure 4: Trajectories naming in the machine properties	6

Figure 5: Trajectories naming in the Initialization function	7

Figure 6: Parameter to choose the splitting strategy in the machine properties	7

Figure 7: Parameter to choose the splitting strategy in the machine properties	8

Figure 8: Help for the strategy choice	8

Figure 9: Example of a setting with the strategy 2	9

Figure 10: Set the splitting comments up	9

Figure 11: Comment to ignore in the initialization function	10

Figure 12: Ignore the Memory in the machine properties	10

Figure 13:  Ignore the Memory in the initialization function	11

Figure 14: sub programs to ignore in the machine properties	11

Figure 15: Sup programs to ignore in the initialization function	12

Figure 16 : Example of empty Ws	13

Figure 17: Begin / End tag	14

Figure 18: Begin/End tag modification	14